System Integration, Communications, AI Pipeline, and Wiring for Perception System

# 1. Executive summary

This document specifies the end-to-end hardware and software integration for a 360° collision-avoidance and inspection using  hexacopter configuration. It details physical connections and serial/UART/CSI/USB mappings, communication protocols, message types and rates, the AI perception stack for LiDAR and stereo cameras, the voxel/ESDF mapping and kinodynamic MPC planner, timing and latency budgets, and comprehensive calibration, testing, and deployment steps. The SIYI ZR30 optical zoom is used exclusively for manual/automated inspection and streams to the GCS

# 2. System

- Sensors:

  - S2 LiDAR (2D) → Serial/USB → Jetson

  - Servo (PWM) → Jetson MCU / PWM controller (for LiDAR rotation control)

  - Stereo Cameras (IMX219) × 8 (4 stereo pairs) → Jetson CSI lanes / USB3 (if needed)

  - SIYI ZR30 gimbal camera → Gimbal controller → Jetson or Cube+ / GCS (RTSP)

  - GNSS #1 → Cube+ UART1 (primary)

  - GNSS #2 → Cube+ UART2 or CAN (redundant)

  - IMU → Cube+ (internal), Jetson receives attitude from Cube+ via MAVLink

- Compute: Jetson Orin Super Nano (perception, mapping, planning) ↔ Cube+ (flight control) via MAVLink (UART4 @ 921600 or Ethernet/UDP)

- Telemetry Radio ↔ Cube+ UART3 → GCS (Mission Planner/QGroundControl) over MAVLink

- Power distribution: Battery → Power module (voltage/current measurement) → ESCs & DC-DC regulators for 5V/12V rails (Jetson, cameras, Cube+, LiDAR, servo, gimbal)

# 3. Physical connections & wiring (detailed)

Important: wire lengths, shielding, and grounding must follow best practices. Use shielded twisted pair for serial lines if long, ferrite beads for EMI mitigation near high-power cables, and secure connectors (M12/Molex/XT60 as suitable).

## 3.1 Power architecture

- Battery (6S nominal, 22.2 V) → Main power distribution board (PDB) → ESCs (motors) and DC-DC converters (for electronics).

- Power module (inline) → Cube+ power input (reports voltage & current via MAVLink).

- DC-DC converters:

  - 22.2 V → 12 V regulated rail: powers Jetson (if required), gimbal, some sensors.

  - 22.2 V → 5 V regulated rail: powers Cube+ (if not using power module direct), cameras (if USB powered), some sensors.

- Polarity/backup: Ensure main battery positive/negative busbars are clearly marked and fused.

## 3.2 Flight controller (Cube+) physical ports

- Power & telemetry:

  - Power-in: from PDB / power module (safe V + current sense to FC).

  - UART1 (TELEM1): GNSS #1 (UBX / NMEA) — TTL UART. Baud: 115200.

  - UART2 (TELEM2): GNSS #2 or secondary telemetry — TTL or CAN (if GNSS over CAN). Baud: 115200.

  - UART3 (RADIO): Telemetry radio (433/915 MHz) — TTL UART. Baud: 57600 or 115200 (match radio); modern radios can do 57600 for robust comms or higher if link supports.

  - UART4 (COMPANION): Companion computer (Jetson) interface — MAVLink v2. Baud recommended: 921600 for high-bandwidth, low-latency. If using UDP: companion Ethernet → Cube+ via companion link is possible (MAVLink over UDP).

- CAN: optional for CAN GNSS or other CAN devices.

- I/O:

  - PWM outputs: ESCs / servos (ESC signal lines via PWM / DShot depending on ESC).

  - AUX ports: for gimbal trigger / camera control if integrated.

## 3.3 Companion computer (Jetson) physical ports

- Power: 12 V regulated → Jetson power module (or 5V depending on board). Use robust DC-DC converter sized for peak Jetson power.

- Camera inputs: Prefer CSI/MIPI for low-latency high-bandwidth. Jetson Orin has multiple CSI lanes — use dedicated multi-camera carrier board or multiplexers. If CSI insufficient use USB3 UVC cameras (bandwidth tradeoff).

- LiDAR: USB or UART depending on S2 model. Use USB3 port if available; otherwise TTL UART to Jetson UART via level shifter.

- Servo control: Jetson GPIO PWM (if available) or separate small microcontroller (e.g., Arduino / Teensy / STM32) receiving commands from Jetson to generate stable PWM for servo; recommended to use a small MCU to offload real-time servo PWM.

- Gimbal: SIYI ZR30 controller connects via UART or Ethernet; Jetson can send commands (pan/tilt/zoom) via the gimbal vendor protocol or via MAVLink GIMBAL messages if integrated. Gimbal video out via RTSP or direct HDMI/SDI to Jetson capture card if required.

- Cube+ connection: Jetson → Cube+ via UART4 (level-shifted TTL serial) or via companion Ethernet → Cube+ UDP. Use MAVLink v2

## 3. 4 Sensor wiring summary table

| Component | Physical link | To | Protocol | Baud/ Rate | Notes |
| --- | --- | --- | --- | --- | --- |
| GNSS #1 | TTL UART | Cube+ UART1 | NMEA/UBX | 115200 | Primary GPS supports L1 ,L5,L2 band |
| GNSS #2 | TTL UART/ CAN | Cube+ UART2 / CAN | NMEA/UBX or CAN | 115200 | Redundant supports L1 ,L5,L2 band |
| Telemetry radio | TTL UART | Cube+ UART3 | MAVLink v2 | 57600 / 115200 | To GCS |
| Jetson | TTL UART/ Ethernet | Cube+ UART4 / Ethernet | MAVLink v2 | 921600 (serial) or UDP | Obstacle msgs / VIO pose |
| S2 LiDAR | USB/ TTL UART | Jetson USB/UART | LiDAR vendor serial | 115200–921600 | Servo angle timestamps required |
| LiDAR servo | PWM (5V) | Jetson MCU / PWM controller | PWM | 50 Hz control | Time-sync with LiDAR scans |
| Stereo pairs (×4) | CSI/ USB3 | Jetson CSI lanes / USB3 | MIPI CSI-2 / UVC | up to Gbps | Use grayscale to reduce bandwidth |
| IMU | Internal to Cube+ | Cube+ | SPI/I²C | 400–1000 Hz | FCU primary attitude source |
| SIYI ZR30 gimbal | UART/ Ethernet | Jetson/Cube+/GCS | Vendor protocol / MAVLink GIMBAL | 115200 / Ethernet | Video: RTSP/H.264 stream |
| Power module | I2C / ADC | Cube+ | Power telemetry | — | Reports voltage/current to FCU |

# 4. Communication protocols & message flows (fully detailed)

## 4.1 MAVLink overview (central comms)

- MAVLink v2 is the canonical messaging layer between Cube+, Jetson, Telemetry Radio, and GCS. Use mission planner (Mission Planner or QGroundControl) as GCS.

- Primary message types used:

  - HEARTBEAT — status & heartbeat (FCU/companion).

  - HEARTBEAT/STATUSTEXT — health/warnings.

  - ATTITUDE, GLOBAL_POSITION_INT, LOCAL_POSITION_NED — FCU pose telemetry.

  - DISTANCE_SENSOR — single sensor distances (LiDAR).

  - OBSTACLE_DISTANCE — sectorized obstacle distances (used by FCU for local avoidance).

  - VISION_POSITION_ESTIMATE / VISION_SPEED_ESTIMATE — VIO or Jetson-provided pose/speed estimates.

  - SET_POSITION_TARGET_LOCAL_NED / SET_POSITION_TARGET_GLOBAL_INT — Jetson commanded setpoints/avoidance setpoints.

  - MISSION_* — for mission upload / download (from Mission Planner).

  - GOPRO_CONTROL* or vendor-specific messages for gimbal control if needed; otherwise MOUNT_CONTROL / MAVLink gimbal messages.

  - PARAM_* — parameter read/write.

- Flow:

  - Jetson publishes OBSTACLE_DISTANCE and, when necessary, SET_POSITION_TARGET_LOCAL_NED to request avoidance maneuvers.

  - Cube+ publishes ATTITUDE and LOCAL_POSITION_NED. Jetson subscribes to these for ego-state.

  - Cube+ applies flight laws; ensure Cube+ parameter configuration accepts companion overrides (use GUIDED mode or set the appropriate acceptance of SET_POSITION_TARGET).

## 4.2 Jetson ↔ Cube+ (Companion link)

- Physical: UART4 TTL level or Ethernet. Use 921600 baud for high throughput if serial. For Ethernet use 1 Gbit link and MAVLink over UDP.

- Messages from Jetson to Cube+:

  - OBSTACLE_DISTANCE (sectorized): Jetson calculates an array of distances around vehicle (e.g., 36 sectors at 10° each) and sends it at ~5–10 Hz.

  - DISTANCE_SENSOR for single LiDAR distance in the forward sector (higher rate).

  - VISION_POSITION_ESTIMATE (if Jetson VIO more accurate than GNSS for short-term) at ~10–30 Hz.

  - SET_POSITION_TARGET_LOCAL_NED for direct avoidance commands or guidance velocity targets.

- Messages from Cube+ to Jetson:

  - ATTITUDE and LOCAL_POSITION_NED at 50–200 Hz (depending on FCU config). Jetson uses these for ego-motion compensation.

  - PARAM_VALUE and STATUSTEXT for status/diagnostics.

## 4.3 LiDAR data handling

- LiDAR outputs range readings for a 2D scan line at its internal scan rate (e.g., 10–20 Hz). Jetson reads raw range + intensity + time stamp.

- Servo angle: Jetson or MCU must timestamp servo position for each scanline so transforms into vehicle frame can be done accurately. If servo angle is not timestamped, the resulting pointcloud will have motion distortion.

- Pointcloud assembly:

  - For each LiDAR scanline (range array), tag with servo angle θ(t) → transform polar to local 3D points in Jetson. Combine many scanlines over servo sweep to build 3D cloud.

- Message to Cube+: Jetson sends DISTANCE_SENSOR for critical single-beam distance (forward) and OBSTACLE_DISTANCE for sector summaries (after fusing LiDAR + stereo).

## 4.4 Stereo & camera data handling

- Camera transport: CSI/MIPI preferred for low-latency. Use hardware sync when possible (frame sync line) so stereo images from left-right are tightly time-aligned.

- Image rate & resolution: typical 640×480@30 fps for side/rear; forward pair may be 1280×720@30 fps. Use grayscale capture for stereo matching to reduce bandwidth.

- Stereo computation: Jetson runs optimized stereo network (TensorRT) yielding disparity maps → depth maps. Depth maps are aligned to vehicle frame using camera extrinsics and VIO pose.

- Image detection: YOLOv8 runs on RGB frames producing bounding boxes and class probabilities. These are projected to 3D using depth to produce 3D detections for fusion.

- Message to Cube+: fused obstacles are summarized in OBSTACLE_DISTANCE and SET_POSITION_TARGET as needed.

## 4.5 Telemetry to GCS (Mission Planner)

- Telemetry radio is connected to Cube+ UART3 and to the GCS (laptop/tablet). It forwards MAVLink streams.

- Video streams from Jetson (BEV thumbnails, inspection feed) are separate — either sent over a high-bandwidth telemetry channel (if available) or via a separate long-range video link. For standard MAVLink radios, compress and send low-bandwidth metadata/alerts; stream video via 4G/5G or Wi-Fi/RTSP when within range.

- Mission Planner features used:

  - Upload mission waypoints and geofences (via MAVLink).

  - View telemetry (attitude, battery, GPS, RC RSSI).

  - See obstacle alerts (custom STATUSTEXT or mapped OBSTACLE_DISTANCE) and preview BEV from Jetson (thumbnail jpeg over MAVLink or separate RTSP).

# 5. AI perception pipeline — end-to-end (algorithmic detail)

This section explains the algorithms, data flow, training, and runtime optimizations end-to-end: LiDAR → stereo → fusion → mapping → planning.

## 5.1 Raw data ingestion & sync

- Time sync is critical. Use PTP / NTP for network devices and hardware timestamps for sensor devices where possible. Jetson should maintain a monotonic clock and annotate arrivals with high-resolution timestamps.

- Buffers / ring buffers: create ring buffers for LiDAR scanlines, camera frames, IMU samples, and servo angles. Align using timestamps.

## 5.2 Ego-motion estimation (VIO / VIO+GNSS)

- VIO (VINS-Fusion) runs on stereo + IMU to produce high-rate short-term accurate pose. If GNSS is good, integrate GNSS for global drift correction.

- Role: provide pose(t) for de-warping LiDAR scans and to place stereo depth in world frame.

## 5.3 LiDAR processing

- Preprocess: filter invalid returns, apply intensity thresholding.

- Time + Servo unwarp: convert each 2D scanline using servo angle θ(t) and vehicle pose at timestamp t to produce 3D points.

- Voxelization / downsample: for speed, use an adaptive downsampling (voxel grid of 0.05–0.1 m depending on region).

- Detection: run PointPillars (fast):

  - Pillarize pointcloud (bird’s-eye view grid), extract pseudo-image features, run 2D conv detection.

  - Output: 3D bbox, class, confidence, velocity (if CenterPoint used).

- Semantic segmentation (optional): RangeNet++ to and classify points (tower/pole/vegetation/bird).

Outputs: 3D detections with positions & confidences, per-point labels for painting.

## 5.4 Stereo depth & image detection

- Stereo matching: StereoNet/PSMNet-small. Produce disparity map D(x,y). Apply sub-pixel refinement (soft-argmax or quadratic fit) to get fractional disparities reducing quantization error.

- Depth map: Z(x,y) = f_px * B / D(x,y). Mask low-confidence disparities (matching cost thresholds).

- Image detection: YOLOv8 returns 2D boxes + scores. Run semantic segmentation (BiSeNet) optionally for point painting.

- Project to 3D: Using camera intrinsics & VIO pose, project image detections with depth into 3D coordinates in the vehicle frame.

Outputs: dense depth for sectors, 3D boxes from stereo-assisted 2D detection

## 5.5 Fusion: point-painting & detection consolidation

- Point-painting: augment LiDAR points by projecting camera segmentation/detections into LiDAR point cloud and adding semantic labels. This helps detection networks by giving image-aware points.

- Score fusion: for overlapping LiDAR & stereo detections, fuse using a weighted scheme: LiDAR weight increases with distance and intensity; stereo weight increases with disparity confidence; image detection weight from YOLO score.

- Result: fused obstacle list with class, 3D box, distance & velocity (from tracker), and per-obstacle confidence.

## 5.6 Tracking & prediction

- 3D Kalman / Extended Kalman filter per tracked object estimating position & velocity in local frame. Use constant-velocity or constant-acceleration motion models.

- Data association: nearest-neighbor + IoU matching (3D CBV IoU) with gating based on expected motion.

- Prediction: estimate short-term future positions (1–5 s) and compute TTC (time-to-collision) with the drone’s current trajectory.

## 5.7 Mapping: TSDF → ESDF (Voxblox)

- TSDF: integrate fused pointclouds into a Truncated Signed Distance Field (TSDF) at voxel resolution 0.1 m (or 0.05 m in near field). This yields surface reconstructions.

- ESDF: compute ESDF (distance-to-nearest-obstacle) from TSDF to provide smooth distance fields for planning. Recompute ESDF each map update in the local window (costly but optimized libs exist).

## 5.8 Kinodynamic planning (MPC) using ESDF

- Planner input: current pose & velocity (from VIO + FCU), target waypoint, local ESDF.

- Cost:

  - path tracking (minimize deviation from mission waypoint)

  - control effort (minimize acceleration & jerk)

  - collision penalty (exponential penalty for voxels where ESDF < safety_distance)

- Constraints: max velocity v_max, max accel a_max, jerk limits.

- Solver: small-horizon MPC (1–3 s horizon) running at 10–30 Hz. Use ESDF gradients to push trajectory away from obstacles.

- Output: smooth velocity/position target to Jetson/Cube+; Jetson issues SET_POSITION_TARGET_LOCAL_NED or SET_ATTITUDE_TARGET depending on integration; Cube+ executes closed-loop.

# 6. Communication protocols, message flows, pros & cons

This section describes each communications path in the system, the exact messages and data that flow, and the advantages / disadvantages (latency, reliability, bandwidth, susceptibility to EMI, operational notes).

## 6.1 Jetson ↔ Cube+ (Companion link)

Physical: TTL UART (UART4) at high baud (recommended 921600), or Ethernet (UDP) if you use a companion network.
Logical: MAVLink v2.

Messages Jetson → Cube+:

- OBSTACLE_DISTANCE — fused sectorized obstacle distances (e.g., 36 sectors). Recommended publish rate: 5–20 Hz.

- DISTANCE_SENSOR — direct LiDAR front-beam values at higher rate (10–50 Hz).

- VISION_POSITION_ESTIMATE / VISION_SPEED_ESTIMATE — VIO pose & speed (10–30 Hz) when Jetson VIO is better than GNSS short-term.

- SET_POSITION_TARGET_LOCAL_NED — avoidance setpoints or immediate velocity targets (on-demand, event-driven).

Messages Cube+ → Jetson:

- ATTITUDE (50–200 Hz) — FCU attitude for sensor compensation.

- LOCAL_POSITION_NED (20–50 Hz) — local pose.

- STATUSTEXT, PARAM_VALUE — status & configuration.

Advantages

- Low-latency control path for immediate avoidance commands (921600 serial is fast).

- Well-supported ecosystem (Mission Planner / QGC) integrates readily with MAVLink.

- Clear message types for obstacle reporting and setpoints.

Disadvantages

- UART at 921600 can be affected by EMI and long cable runs — keep cable short and shielded; use twisted pair and ferrites.

- Companion overrides must be carefully gated in Cube+ parameterization to avoid unsafe commands from buggy Jetson code.

- If using UDP/Ethernet, you gain bandwidth but add complexity (network stack, IP addressing, NAT with GCS if remote).

Operational notes

- Use parity/error checking and consider a small watchdog on Cube+ that rejects SET_POSITION_TARGET commands that violate safe limits (max speed/altitude).

- Keep a heartbeat policy: if Jetson heartbeat not seen, Cube+ should disable companion overrides and revert to RTH / mission.

## 6.2 Cube+ ↔ GCS (Telemetry radio)

Physical: Radio module on Cube+ UART3 → GCS (3 km or more link as spec).
Logical: MAVLink v2.

Messages & flows

- Full vehicle telemetry (heartbeat, attitude, position, battery, mission).

- Operator receives obstacle alerts relayed (statustext or custom MAVLink messages).

- Mission upload/download (waypoints, geofence).

- Operator can command SIYI gimbal if linked through Jetson/Cube+.

Advantages

- Standard, proven link for remote control.

- GCS tools (Mission Planner / QGC) parse MAVLink and provide operator UI.

Disadvantages

- Telemetry radios have limited bandwidth — avoid high-bitrate streaming over the same link (do not send raw video).

- Outdoor range and reliability affected by terrain and EMI from HV lines; plan for intermittent dropouts.

Operational notes

- Stream only metadata/alerts over telemetry. For video, use separate high-bandwidth link (Wi-Fi/4G/5G/RTSP over cellular) or low-res thumbnail over MAVLink.

- Auto-RTH on long telemetry dropout should be configured conservatively with operator overrides.

## 6.3 Jetson ←→ LiDAR (S2) & Servo control

Physical: LiDAR → Jetson via USB or TTL serial; servo angle feedback via PWM/encoder to Jetson or MCU.

Messages

- LiDAR → Jetson: raw range array + timestamp + intensity.

- Jetson → servo MCU: PWM commands for angle control (or target angle via serial) and request for angle timestamps (or read encoder).

Advantages

- LiDAR gives precise range returns (cm-level) and works at night.

- Servo + LiDAR arrangement provides 3D by scanning.

- Servo moves 90 - 120 degrees in 2 secs for 3D mapping.

Disadvantages

- LiDAR + servo 3D update rate depends on servo rotation speed — may be slow for high-speed flight if not planned.

- LiDAR sensitive to atmospheric scatter (rain/fog/dust) and reflective surfaces.

Operational notes

- Timestamp LiDAR scanlines and servo angle with microsecond precision. Use a hardware PWM driver or MCU with encoder for reliable angular feedback — do not rely on open-loop servo commands only.

- Provide mechanical slip-ring or cable routing for continuous 360° rotation if needed.

## 6.4 Jetson ←→ Stereo Cameras (IMX219 pairs)

Physical: CSI/MIPI preferred; if insufficient CSI lanes use USB3 UVC cameras.

Messages

- Camera streams → Jetson: frames (grayscale preferred for stereo), per-frame timestamps.

- Jetson runs stereo inference and publishes depth maps and 3D detections to fusion node.

Advantages

- Stereo provides dense depth map in textured scenes and is cheap/low-power.

- Good for classification (bird vs pole) via image detectors.

Disadvantages

- Passive sensor — fails in low light, glare, and low-texture surfaces.

- High bandwidth — manage resolution/FPS tradeoffs.

- Sensitive to motion blur at high speed; use higher shutter speed and more light.

Operational notes

- Use hardware frame sync (if camera modules support) to ensure left-right frames are coherent.

- Prefer grayscale stereo at real-time; reserve RGB at lower rates for detection/classification.

## 6.5 SIYI ZR30 Gimbal → GCS / Jetson

Physical: Gimbal controller ↔ Jetson (UART/Ethernet) and/or direct stream to GCS via RTSP.

Messages

- Gimbal telemetry & control via vendor protocol or MAVLink GIMBAL messages.

- Video streamed via RTSP/H.264 to operator for inspection.

Advantages

- High-quality optical zoom for inspection (20×); good for detailed visual inspection when hovering.

- Operator-driven — reduces demand on autonomy for inspection tasks.

Disadvantages

- Narrow FOV at high zoom; image stabilization needed.

- Mechanical complexity: moving parts risk in dust/wet environments.

Operational notes

- Use gimbal-safe interlocks (do not allow large gimbal motions during aggressive maneuvers).

- Stream video separately from MAVLink (use Jetson as transcode/RTSP server if needed).

# 7. Weather effects on sensors — detailed, quantitative guidance & mitigations

Environmental conditions affect each sensor differently. Below I detail the effects, give practical numeric guidance where feasible, and list mitigations.

## 7.1 S2 2D LiDAR (servo-scanned to 3D)

### How weather affects LiDAR

- Rain, fog, mist: scattering causes false returns and range attenuation; heavy fog significantly reduces effective range and adds noise. Backscatter near sensor can create many spurious returns.

- Snow: similar to fog; many false returns and clutter.

- Dust: reduces SNR and may cause spurious returns.

- Sunlight: direct sun rarely affects time-of-flight LiDAR significantly, but certain low-cost LiDAR units can pick up sunlight-induced noise.

- Surface reflectivity: very dark (low albedo) or transparent objects (glass) may yield weak/no return.

### Practical impact examples (conceptual)

- In light rain/fog, effective reliable range can shrink by 30–70% depending on severity and LiDAR power/wavelength.

- In heavy fog, effective detection range may be reduced to only a few meters — not reliable for high-speed avoidance.

### Mitigations

- Sensor fusion: rely on stereo (in good light) and radar (if available) when LiDAR confidence drops. Implement a sensor-confidence measure and dynamic speed limiter.

- Use return intensity & multi-echo: use intensity to filter low-confidence returns; if LiDAR supports multi-echo, use secondary echoes to detect partially obscured obstacles.

- Sector prioritization: during fog, reduce speed and increase updates in the forward sector; enlarge safety margins.

- Physical protection: hydrophobic lens covers, heated windows/anti-fog coatings minimize water droplets effects.

- Alternative tech: for heavy-fog ops consider mmWave radar (radar less affected by fog/rain but lower resolution).

## 7.2 Stereo cameras (IMX219 pairs)

### How weather affects stereo

- Low light/night: stereo matching performance degrades due to reduced texture and SNR; disparity becomes noisy or fails.

- Rain / droplets on lens: local distortions and specular highlights create false matches; raindrops blur images.

- Fog / haze: reduces contrast and texture; stereo matching fails at longer ranges.

- Snow: similar to rain/ fog; high-contrast flakes create false features and moving clutter.

- Sun glare and backlight: saturation and reduced usable image regions.

### Practical impact examples

- At twilight / low-illuminance, stereo matching error increases; effective reliable range reduces often by 50% or more unless additional illumination (IR) is provided.

- Motion blur with low shutter speed (in low light) reduces matching quality — use higher shutter speed but accept higher sensor gain (noise).

### Mitigations

- IR illumination: add NIR illuminators for stereo; airborne use must consider wildlife and regulations. Many camera sensors are NIR-sensitive; use IR-pass filters as needed.

- Use larger aperture / tele lenses for forward pairs: increases photons per pixel, helps low-light; tradeoff narrow FOV.

- HDR & adaptive exposure: develop AE algorithms that avoid saturation. Use per-camera exposure control to avoid overexposure in high-contrast scenes.

- Hydrophobic lens coatings and wipers: keep optics clear during light rain.

- Subpixel & temporal fusion: use temporal smoothing and motion compensation from IMU to improve stereo in low SNR.

- Fallback to LiDAR / radar in severe conditions.

## 7.3 SIYI ZR30 (20× optical zoom) — inspection camera

### How weather affects optical-zoom

- Atmospheric turbulence: long optical paths make zoomed images susceptible to scintillation and blur (heat shimmer), especially over tens of meters.

- Low light: narrow aperture at high zoom reduces light per pixel — image SNR drops; camera auto-ISO may amplify noise.

- Rain / water droplets: at high zoom, a droplet on lens massively distorts view and obscures target.

- Fog / haze: contrast loss and scattering severely reduce detail at range — long-range inspection in fog is very poor.

- Wind / vibration: higher zoom magnifies image motion; gimbal stabilization needs to be effective.

### Mitigations

- Use gimbal stabilization and image stabilization algorithms (on-gimbal + software).

- Use two-stage imaging: wide-angle pass for approach, zoom-in only when within known safe standoff.

- Use hydrophobic coatings and lens heaters (small heating element) to reduce condensation/fogging in humid/cold conditions.

- Prefer daytime, low-humidity windows for detailed inspection; for night inspections use powerful zoom + auxiliary lighting (but light may be limited).

## 7.4 Combined operational policy (sensor-confidence & speed limits)

- Build a sensor-confidence metric per sensor (LiDAR_conf, Stereo_conf, GNSS_conf) based on intensity, SNR, disparity-cost, satellite count, HDOP.

- Compute effective detection range = weighted fusion of sensor ranges and confidences.

- Implement dynamic speed cap: limit cruise speed such that stopping distance at current speed < effective detection range minus margin. Example policy:

  - If effective detection range < stopping_distance * 1.2 → reduce speed proportionally or land / RTH.

# 8. Latency, rates & timing budgets (practical targets)

Aim to keep end-to-end perception-to-command latency ≤ 200 ms. Lower is better.

## 8.1 Example budget (target)

- Camera capture & transfer to Jetson: 10–20 ms (CSI).

- Stereo depth inference (per prioritized pair): 20–50 ms (TensorRT FP16).

- Image detection (YOLOv8 tiny) per frame: 10–30 ms.

- LiDAR read + servo timestamp: 10–30 ms per scanline; assembly of partial cloud: 20–50 ms depending on sweep.

- Fusion & mapping update (TSDF/ESDF incremental): 20–40 ms (optimized).

- Tracking & prediction: <10 ms.

- MPC planning & setpoint generation: 10–30 ms.

- MAVLink transfer to Cube+: <10 ms (serial at 921600).

- Cube+ actuation + control loop response: ~50 ms (to start applying control).

Total ~ 160–270 ms depending on concurrency. Profile and reduce by parallelism and CPU/GPU optimization.

## 8.2 Message rates suggestions

- ATTITUDE: 50–100 Hz from Cube+.

- LOCAL_POSITION_NED: 20–50 Hz.

- OBSTACLE_DISTANCE: 5–20 Hz (more frequent if obstacle density high).

- DISTANCE_SENSOR: 10–50 Hz for direct LiDAR front beam.

- Jetson internal processing loops: perception @ 10–30 Hz (full fused updates), stereo forward pair faster @ 30 fps if possible for forward reaction.

# 9. Calibration & alignment (must-have steps)

- Intrinsic camera calibration — for each IMX219 camera: focal length, principal point, distortion coefficients. Use checkerboard or chessboard calibration. Save YAML intrinsics.

- Stereo extrinsics — left-right transform (R, t) per stereo pair. Use stereo calibration tools (OpenCV stereoCalibrate) and refine with bundle-adjusted sequences.

- Camera ↔ LiDAR extrinsics — rigid transform between LiDAR and one camera frame. Use calibration rigs or automatic calibration (e.g., by detecting same features in both modalities). Record and verify.

- LiDAR servo mapping — verify servo angle to physical transform mapping: angle → rotation axis calibration to ensure accurate pointcloud alignment.

- IMU ↔ camera / IMU ↔ LiDAR — calibrate time offset and transform. IMU offset critical for VIO accuracy.

- GNSS → local frame alignment — verify offsets and correct NED/ENU conventions.

- Jetson ↔ Cube+ time sync — configure time sync (NTP/PTP) or use MAVLink timestamps conversion.

- Motor & ESC calibration — calibrate throttle endpoints, failsafe throttle.

- Gimbal calibration — ensure SIYI zero positions, zoom limits, and RTSP stream validated.

# 10. Safety, failsafe, and degradation modes

## 10.1 Mandatory failsafes

- Dual GNSS: automatic switchover. If both GNSS degrade, Jetson triggers VIO-only navigation within limited area and reduces speed.

- Companion failure: if Jetson fails/unavailable, Cube+ uses preplanned contingency: reduce speed, enter fail-safe loiter, and perform RTH if necessary.

- Telemetry loss: auto RTH and safe loiter in case of long telemetry blackout.

- Low battery: Cube+ to enforce RTL when battery below threshold with return margin.

- Obstacle detection overrides: Jetson-issued avoidance has high priority but Cube+ should have a safety veto (e.g. prevents Jetson from issuing commands that violate hard constraints like exceed motor limits). Use parameter COM_OBL_ACT style mapping.

## 10.2 Graceful degradation

- Night/Low light: Jetson detects low stereo confidence and increases weighting of LiDAR; automatically reduces v_max.

- Heavy rain/fog: LiDAR returns confidence drop — increase safety buffer and force slow speed / land if necessary.

- High EMI near power lines: GNSS may degrade — rely on VIO and ensure operator notified via GCS.

# 11. Training datasets, labels & example pipelines (practical)

## 11.1 LiDAR dataset

- Schema: PCAP-like rosbags or custom binary with synchronized frames: LiDAR scans + servo angle + IMU + camera frames + GNSS.

- Labels: 3D bounding boxes (class labels: tower, pole, conductor segment, bird, vehicle, human). Use tools: LabelCloud, Scalabel, or KittiLabelTool.

- Size: Thousands of annotated frames across environmental conditions; include synthetic augment.

## 11.2 Stereo / image dataset

- Schema: Left-right images + synchronized depth (sparse LiDAR) + intrinsics/extrinsics.

- Labels: 2D boxes, segmentation masks of features (tower parts, insulators, birds).

- Validation: Edge-case test set for night, glare, snow/rain.

## 11.3 Training workflow

- Pretrain on KITTI / NuScenes for base detectors.

- Fine-tune on domain dataset (powerline scenes).

- Use cross-validation & hold-out test sets.

- Deploy: export models to TensorRT FP16; verify on Jetson with representative inputs.

# 12. Testing, validation & acceptance plan

## 12.1 Bench tests (lab)

- Sensor connectivity and serial comms checks.

- Time-sync verification across Jetson, Cube+, LiDAR.

- Run inference pipelines on Jetson offline using recorded data; verify latency and resource usage.

- Verify MAVLink message handling between Jetson & Cube+ (setpoints, obstacle messages).

## 12.2 Indoor flight tests

- Controlled environment (Vicon or motion-capture): test slow-speed avoidance (1–3 m/s) against static obstacles and moving obstacles (pushed on rails). Validate end-to-end loop.

## 12.3 Outdoor progressive tests

- Phase 1: Low altitude, low speed (≤5 m/s). Validate LiDAR + stereo detection, tracking, avoidance.

- Phase 2: Increase speed to 10 m/s, test forward sector scan rates and stereo tele-pair detection. Tune MPC parameters and sensor confidence thresholds.

- Phase 3: Higher speed if long-range sensor added. Test failsafes, GNSS near towers, EMI robustness.

## 12.4 Edge condition validation

- Night flights to verify LiDAR redundancy & stereo low-light behavior.

- Fog/rain simulation if safe.

- GNSS-denied flights to verify VIO fallback.

## 12.5 Acceptance criteria

- Obstacle detection precision / recall above thresholds at operational ranges (e.g., >95% detection for >10 m within 0–10 m obstacle class).

- End-to-end avoidance success rate >99% in trial scenarios.

- Latency <200 ms for typical avoidance sequence.

- Safe RTH & failover validated.

# 13. Power & flight-time summary (concise, for your Hex ~7.5 kg)

## 13.1 Baseline values used

- Electronics power ~ 90 W.

- Two propulsion efficiency cases:

  - Case A: 130 W/kg → propulsion ≈ 975 W → total ≈ 1,065 W.

  - Case B: 100 W/kg → propulsion ≈ 750 W → total ≈ 840 W.

## 13.2 Flight time table (5k/10k/15k/20k mAh, 6S = 22.2 V, usable 85%)

- Case A (Total 1,065 W):

  - 5,000 mAh → ≈5.3 min

  - 10,000 mAh → ≈10.6 min

  - 15,000 mAh → ≈16.0 min

  - 20,000 mAh → ≈21.3 min

- Case B (Total 840 W):

  - 5,000 mAh → ≈6.7 min

  - 10,000 mAh → ≈13.5 min

  - 15,000 mAh → ≈20.2 min

  - 20,000 mAh → ≈27.0 min

## 13.3 For 40 minutes endurance estimate

- Expect ~30k–45k mAh at 6S depending on propulsion efficiency and energy density — iterative sizing required (see previous detailed iterative results). Battery mass is significant and must be iteratively included in total mass.

# 14. Practical recommendations & implementation checklist

- Time sync: Implement hardware timestamping; Jetson and Cube+ must have synced clocks.

- Servo-LiDAR synchronization: timestamp servo position with each LiDAR line.

- Companion link: use 921600 serial or UDP to minimize latency; configure FCU to accept companion overrides.

- Pilot-in-loop: for inspection flights with SIYI ZR30, maintain operator control; inspection camera not used as primary obstacle sensor.

- Sector prioritization: process forward sector at highest rate and fidelity.

- Dynamic speed limiter: implement on Jetson — if perceived safe stopping distance > available stopping distance, reduce v_max.

- Model optimization: convert to TensorRT FP16; prune backbones.

- Logging: log synchronized raw sensor data and events for offline tuning & model improvements.

- EMI tests: verify flights near high-voltage transmissions to see GNSS degradation & telemetry robustness.

# 15 Appendices

## Appendix A — MAVLink key message mapping (quick)

- OBSTACLE_DISTANCE — sectors array, available obstacles distance (publish at 5–10 Hz).

- DISTANCE_SENSOR — specific LiDAR / sonar.

- VISION_POSITION_ESTIMATE — local pose from Jetson VIO.

- SET_POSITION_TARGET_LOCAL_NED — Jetson commanded avoidance setpoint.

- HEARTBEAT, ATTITUDE, LOCAL_POSITION_NED — FCU telemetry.

## Appendix B — Example OBSTACLE_DISTANCE usage

- Sectors: choose 36 sectors (10° each) or 72 (5°). Populate with fused min distance per sector; set increment field appropriately; Message frequency 5–10 Hz.

## Appendix C — Glossary

- VIO: Visual-Inertial Odometry

- TSDF: Truncated Signed Distance Field

- ESDF: Euclidean Signed Distance Field

- MPC: Model Predictive Control

- TTC: Time To Collision

Calculations

## Numeric worked examples (three commonly used angular resolutions)

I compute for S = 360, 720, 1440 and F = 15 Hz (also show F = 20 and 40 where helpful). Key outputs: pps, B/s, points in 3 s and 5 s windows, points per map tick at 10 Hz and 15 Hz, and memory to hold T window.

### 1) S = 360 samples/scan; F = 15 Hz

- pps = 360 × 15 = 5,400 pts/s

- B/s = 5,400 × 16 = 86,400 B/s = 0.0864 MB/s (≈ 0.69 Mb/s)

- Points in T=3 s → 5,400 × 3 = 16,200 pts

  - Memory to store those points = 16,200 × 16 = 259,200 B ≈ 0.259 MB

- Points in T=5 s → 5,400 × 5 = 27,000 pts → memory = 27,000 × 16 = 432,000 B ≈ 0.432 MB

- points_per_tick at R_map=10 Hz → 5,400 / 10 = 540 pts/tick

- points_per_tick at R_map=15 Hz → 5,400 / 15 = 360 pts/tick

### 2) S = 720 samples/scan; F = 15 Hz (typical mid-density)

- pps = 720 × 15 = 10,800 pts/s

- B/s = 10,800 × 16 = 172,800 B/s = 0.1728 MB/s (≈ 1.38 Mb/s)

- Points in T=3 s → 10,800 × 3 = 32,400 pts → memory = 32,400 × 16 = 518,400 B ≈ 0.518 MB

- Points in T=5 s → 10,800 × 5 = 54,000 pts → memory = 54,000 × 16 = 864,000 B ≈ 0.864 MB

- points_per_tick at 10 Hz → 10,800 / 10 = 1,080 pts/tick

- points_per_tick at 15 Hz → 10,800 / 15 = 720 pts/tick

### 3) S = 1440 samples/scan; F = 15 Hz (high-density)

- pps = 1440 × 15 = 21,600 pts/s

- B/s = 21,600 × 16 = 345,600 B/s = 0.3456 MB/s (≈ 2.76 Mb/s)

- Points in T=3 s → 21,600 × 3 = 64,800 pts → memory = 64,800 × 16 = 1,036,800 B ≈ 1.037 MB

- Points in T=5 s → 21,600 × 5 = 108,000 pts → memory = 108,000 × 16 = 1,728,000 B ≈ 1.728 MB

- points_per_tick at 10 Hz → 21,600 / 10 = 2,160 pts/tick

- points_per_tick at 15 Hz → 21,600 / 15 = 1,440 pts/tick

## Voxel (TSDF) mapping: How many voxels get created/updated?

We estimate unique voxels in the T-second map snapshot and voxel updates/sec the mapping must perform.

- unique_voxels_in_T ≈ (points_in_T) × f_uniq

- voxel_memory ≈ unique_voxels_in_T × bytes_per_voxel (assume 16 B/voxel)

- voxel_updates_per_second ≈ pps × k, where k is the average number of voxels a single point updates (depends on TSDF truncation radius). Conservative k ≈ 8 (voxel neighborhood touched per integrated ray); pick 8 as baseline.

Using f_uniq = 0.6 and k = 8 (conservative for 2D-lidar-derived clouds):

### Example (S = 720, F = 15) — mid-density (most realistic)

- points_in_3s = 32,400 → unique_voxels ≈ 32,400 × 0.6 = 19,440 voxels

  - voxel memory ≈ 19,440 × 16 B = 311,040 B ≈ 0.311 MB

- points_in_5s = 54,000 → unique_voxels ≈ 54,000 × 0.6 = 32,400 voxels

  - voxel memory ≈ 32,400 × 16 = 518,400 B ≈ 0.518 MB

- voxel_updates_per_second ≈ pps × k = 10,800 × 8 = 86,400 writes/sec

- voxel_updates_per_tick (R_map = 10 Hz) = 86,400 / 10 = 8,640 writes/tick

  - (At 15 Hz → 86,400 / 15 = 5,760 writes/tick.)

### Example (S = 360, F = 15)

- points_in_3s = 16,200 → unique_voxels ≈ 9,720 → mem ≈ 155,520 B (0.156 MB)

- voxel_updates/sec ≈ 5,400 × 8 = 43,200 → per tick @10Hz = 4,320 writes

### Example (S = 1440, F = 15)

- points_in_3s = 64,800 → unique_voxels ≈ 38,880 → mem ≈ 622,080 B (0.622 MB)

- voxel_updates/sec ≈ 21,600 × 8 = 172,800 writes/sec → per tick @10Hz = 17,280 writes

Mapping update rate (10–15 Hz)

Mapping update (10–15 Hz): incremental integration — each tick you process new points (the points_per_tick batch) and update TSDF & ESDF locally. This gives you a continuously updating map at 10–15 Hz.

## Practical recommendations & knobs you should tune now

- Use sparse hashed TSDF (Voxblox) — minimal memory & very fast for LiDAR-only mapping.

- Keep R_map = 10–15 Hz as you specified; for S=720 & F=15 each tick processes only a few thousand voxel writes — feasible.

- Use encoder/servo timestamps to correctly place points in world frame before insertion. Even though points/sec is modest, timestamp accuracy matters for moving drone.

- Cap TSDF truncation radius to limit k (voxels updated per point) — smaller truncation reduces writes. For v=0.1 m, use truncation 0.2–0.3 m.

- Limit ESDF recompute radius to local band (e.g., 10–15 m) rather than global recompute each tick.

- Log only TSDF snapshots or small pointcloud extracts for offline debugging — raw LiDAR logs are tiny but full-rate continuous logging is usually unnecessary.

- Profile early: measure your actual S and F and use these numbers to size exact CPU/GPU kernels. The theoretical numbers above are small; optimizations will make it smooth.

Quick overview & hardware wiring

- Mount CS20 facing straight down (optical axis aligned with vehicle Z). If it is offset, record the offset & set PLND_ORIENT (mount orientation) later. CS20 is typically a USB/ToF camera — connect it to your companion computer (e.g. Raspberry Pi) over USB.

- Companion computer (Pi/Jetson) runs code to:

  - convert CS20 depth frames → location of a safe landing target (x,y relative to camera + distance z), and

  - publish MAVLink LANDING_TARGET (and optionally DISTANCE_SENSOR) to the autopilot. ArduPilot uses the LANDING_TARGET message for precision landing.

- Flight controller (Pixhawk) connected to companion via TELEM port or USB. Mission Planner will show messages in MAVLink inspector when streaming correctly.

# 1. ArduPilot / Mission Planner parameters to set

Set these (Mission Planner → Full Parameter List). After changing parameters reboot FCU.

- PLND_ENABLED = 1 (enable Precision Landing).

- PLND_TYPE = 1 (use MAVLink LANDING_TARGET from companion computer).

- PLND_SRCH_ALT — search altitude (m) to climb to if target lost (start with 6–10 m).

- PLND_ALT_MAX — max altitude to use precision landing (final stage).

- PLND_STRICT — 0 (fallback to normal land if target lost) or 1 (abort /strict). Start with 0.

- PLND_ORIENT — set if camera is not mounted with x-forward, y-right.

- RNGFND_TYPE / RNGFND_SCALING / RNGFND_MIN_CM / RNGFND_MAX_CM — if you provide DISTANCE_SENSOR messages from CS20, configure rangefinder settings or let ArduPilot auto-detect. Downward rangefinder data improves altitude control during final descent.

(Exact param names above are ArduPilot parameter names; check Mission Planner parameter reference before flight.)

# 2. What the companion computer must send

- LANDING_TARGET (MAVLink) — contains either:

  - angular offsets (angle_x, angle_y) + distance OR

  - absolute x,y,z in MAV_FRAME_LOCAL_NED (position_valid=1). ArduPilot supports both; sending x,y,z in local frame is best if you can compute it. Send at 10–50 Hz. mavlink.io+1

- DISTANCE_SENSOR (MAVLink) — publish the measured height above ground from CS20 as a rangefinder input (helps FCU altitude). Send at ~10–20 Hz

Important: during final approach ArduPilot will accept external target corrections (precision loiter/landing). If target is lost it will either continue normal landing or abort depending on PLND_STRICT.

# 3. Minimal Python (companion computer)

PyMAVLink. It includes:

- connecting to FCU (UDP or serial),

- placeholder code to read CS20 depth frames (I mark the place to call your CS20 SDK/ROS node),

- simple algorithm: pick the largest flat/clear circle near center as "safe spot", compute angle offsets and distance, and send LANDING_TARGET + DISTANCE_SENSOR.

Notes before running: install pymavlink, opencv-python, and the CS20 SDK / ROS driver you use. Adjust FCU_CONNECTION to your link (e.g. /dev/ttyAMA0, udp:127.0.0.1:14550, or serial). This script is intentionally straightforward so you can adapt the detection step to your preferred depth-processing method (ROS, OpenCV, pointcloud libs).

How it works (summary):

- find_safe_spot() is minimal: replace with a plane fit + obstacle detection (RANSAC plane, normal variance, slope threshold) for production. If you can detect flat area and no obstacles within the rotor downwash radius, that's a good safe spot.

- Convert pixel offsets → angle_x/angle_y or compute absolute x,y,z in local NED (more precise if you can). ArduPilot accepts both when PLND_TYPE=1.

# 4. Practical tips, safety & testing

- Simulate first: use SITL and simulate LANDING_TARGET messages before trying on real vehicle. ArduPilot docs explain precision landing workflows.

- Start high PLND_SRCH_ALT so vehicle can search for target on approach if it isn’t seen. Test PLND_STRICT=0 initially (fallback to normal land).

- Rate: send LANDING_TARGET at ≥ 5–10 Hz; 10–50 Hz recommended for smooth corrections.

- Rangefinder: sending DISTANCE_SENSOR helps the autopilot use the camera height as a reliable altitude source for final descent (especially if barometer/GPS noisy).

- Yaw alignment: if you want a particular yaw on touchdown, compute and send yaw through the landing target quaternion fields (advanced) or align yaw before entering Land mode. Some FC firmware variations handle yaw differently — test on bench.

- Prop downwash & turbulence: final few meters are sensitive — ensure landing zone has enough clearance and that your large-rotor turbulence won’t move the vehicle off the pad. Consider using a bigger landing pad or net if turbulence is an issue.

- Logging & MAVLink inspector: view received LANDING_TARGET in Mission Planner’s MAVLink Inspector to confirm the FCU sees your messages.

## SETUP SUMMARY (for clarity)

You’re using:

- CS20 LiDAR (downward-facing, via USB-C → Raspberry Pi / Jetson)

- Companion computer sends LiDAR range to Cube+ via MAVLink (RNGFND messages)

- Used for safe spot landing, possibly assisted by camera or vision system.

## ADVANTAGES

### 1. High-Accuracy Altitude Data

- CS20 uses Time-of-Flight (ToF) — accurate to a few centimeters.

- Great for low-altitude hover and final descent (0–5 m range).

- Better precision than barometer or GPS-based altitudes.

### 2. USB Interface = Noise-Free & Plug-and-Play

- USB-C isolates data well from power noise (unlike UART cables that pick up PWM interference).

- No baud rate or wiring mismatch issues once connected to the companion computer.

### 3. Easy Integration with Vision/AI on the Same Device

- If your companion computer also runs a downward camera (for ArUco tags, pad detection, etc.), both camera and LiDAR data can be fused in software before sending to Cube.

- You can combine LiDAR distance + camera detection to confirm a “safe spot.”

### 4. Flexible Data Fusion & Filtering

- On the Pi/Jetson, you can apply filters (e.g., Kalman, moving average) or even ignore bad readings (e.g., reflections on shiny surfaces).

- You can even train a small ML model to predict if a surface is flat/safe to land based on LiDAR variance + image texture.

### 5. Can Work Beyond ArduPilot Limits

- You’re not limited to the flight controller’s built-in RNGFND logic.

- You can design custom landing conditions (e.g., “Land only if variance < 0.1 m for 3 s” or “Abort if slope detected”).

### 6. Mission Safety & Redundancy

- Having a companion computer gives an extra safety layer.

- If GPS or barometer drifts, LiDAR maintains precise local height for a soft touchdown.

## FLAWS / LIMITATIONS

### 1. Latency (Communication Delay)

- Data path: LiDAR → Pi → MAVLink → Cube → Landing Controller.

- Adds ~100–200 ms delay, depending on serial and script efficiency.

- In fast descents or gusty conditions, delayed altitude updates can cause bounces or early contact.

Mitigation:

Use high MAVLink rate (10–20 Hz), lightweight Python/C++ code, and low serial latency.

### 2. Dependency on Companion Computer

- If the Pi/Jetson crashes, reboots, or disconnects, the LiDAR data stops.

- The Cube+ then loses terrain feedback and might assume altitude = 0 m.

Mitigation:
Add watchdog logic: Cube ignores data loss for a few seconds or aborts landing safely.

### 3. USB Power & EMI Sensitivity

- USB-C LiDAR draws more power (≈ 400–600 mA).
If powered from the Pi’s USB bus and the Pi browns out under load (e.g., camera + LiDAR + Wi-Fi), LiDAR drops connection mid-flight.

Mitigation:
Use a powered USB hub or a dedicated 5 V regulator (2 A) for LiDAR.

### 4. USB Driver/Kernel Issues

- Linux USB ports occasionally reset due to EMI, cable length, or hub instability.

- Unlike UART sensors, USB devices can “disconnect” and re-enumerate mid-flight.

Mitigation:
Use short shielded cable, high-quality hub, and udev rules to lock serial port IDs.

### 5. Cannot Interface Directly to Cube+

- If the Pi fails, there’s no direct backup — Cube alone can’t read USB-C LiDAR.

- Contrast: a TFmini UART LiDAR could still provide direct fallback data.

Mitigation:
Optional: add a second, lightweight UART LiDAR (e.g., TFmini-S) purely as a backup.

### 6. Weather / Surface Reflection Issues

- LiDAR (especially IR-based) can misread:

  - Shiny or reflective surfaces (metal roofs, puddles)

  - Transparent materials (glass, water)

  - Dark asphalt (low reflectivity)

Mitigation:
Use sensor fusion (LiDAR + vision) to confirm surface confidence before landing.

### 7. Limited Field of View (Single Spot)

- CS20 gives only one distance point (center beam).
It doesn’t map an area or detect if the nearby surface is sloped or uneven.

Mitigation:

- Use multi-scan descent (hover a few cm above, sample around with drone motion).

- Or fuse with downward-facing depth camera or stereo pair for slope estimation.

### 8. Mission Planner Integration Is Indirect

- Since altitude data comes via MAVLink, Mission Planner treats it as an external rangefinder — you can’t access advanced “LiDAR landing” features like you would with a natively supported UART LightWare.

Mitigation:
Custom LUA script or onboard companion logic triggers landing via GUIDED mode.